+------------------------------------------------------------+

|     Dynamic Mouse Driver Release Notes (version 9.10)      |

|     Copyright (C) KYE Systems Corp. 1992                   |

+------------------------------------------------------------+

The following mouse parameters have been added to the MOUSE

driver which allow you to configure the mouse or trackball to

fit your requirements.

A. Communication Port ( p )

------------------------

Your mouse can be connected through a PS/2 mouse port, a port

on a mouse bus extension card, or a serial (COM) port. If

connected to either of the first two types of ports, the mouse

driver will automatically detect and configure it when you

load the MOUSE driver. If connected to a COM port other than

COM1, you must inform the MOUSE driver when loading it.

The command format is:

MOUSE p    (Where p = 1 to 4).

For example, if you install the mouse at COM2 (default

setting), enter the following when loading the driver:

MOUSE 2

However, for loading the driver when the mouse is connected

to COM1, a bus mouse port, or a PS/2 port, no parameter is

needed. Type:

MOUSE

to load the driver.

The standard Relative Address and IRQ no. for each COM port

are listed in the table below:

-------+----------+------------------+----------

port |  COM no. |  Address in HEX. |  IRQ no.

-------+----------+------------------+----------

1   |  COM 1   |       3F8        |  IRQ 4

2   |  COM 2   |       2F8        |  IRQ 3

3   |  COM 3   |       3E8        |  IRQ 4

4   |  COM 4   |       2E8        |  IRQ 3

-------+----------+------------------+----------

These settings should be acceptable for each COM port. However

if there is a need to set a specific I/O port - IRQ no. for a

given port, refer to section C.

B. Acceleration Setting ( *a )

---------------------------

The acceleration parameters determine the relative speed of

cursor movement for set distances of mouse movement within a

standard amount of time. On the existing five acceleration

settings, speed is slowed for shorter distances and increased

for longer distances. Thus, when operating your mouse, you get

more precision in short distances, but are still able to move

the cursor across the whole screen with a quick, short movement.

You can choose from five acceleration settings. Setting 1

is a slower setting, Setting 5 is relatively fast.

The default is Setting 3. To choose another setting, type:

MOUSE *a   (Where a = 1 to 5, if omitted, Setting 3 is selected.)

i.e.: MOUSE *2 selects Acceleration Setting 2.

C. I/O Port Address & IRQ no. ( -i,q )

-----------------------------------

This feature is provided for experienced users. If you're

unfamiliar with I/O ports and IRQ channels, don't attempt to

change these because any mistakes could prevent you from

using your computer. In general these settings do NOT need

to be changed. However, if your COM port is not set to any of

the standard I/O Port Addresses and IRQ combinations listed in

the table under section A, you need to directly tell the MOUSE

driver the I/O Port Address and IRQ No. when loading the driver.

To select a non-standard I/O Port Address and IRQ channel,

type:

MOUSE -i,q (that is "dash""I/O Port Address""comma""IRQ No.")

The IRQ setting can range from 2 through 5. The default is 5.

Example:

MOUSE -3E0,2 (Loads the driver, selects 3E0H port address

and IRQ no. 2)

WARNING: If the wrong setting is selected or not set

properly, you may cause system error!

D. Switch Ortho Line Function  ( +s )

----------------------------------

The Ortho Line function is provided as an aid to the user

for drawing straight horizontal or vertical lines. You can

set this function to be ON or OFF, by setting the parameter

to "+1" or "+0". The default for this function is OFF.

Once you have set the Ortho line function "ON", you press

the left and right button of the mouse (or trackball) at the

same time to start the function. Use the same method to

turn the function "OFF" when you don't want to use it.

Then, to activate the Ortho Line Function, type the command:

MOUSE +1   (Switches the Ortho Line Function ON)

Then simultaneously press the left and right buttons.

Pressing the buttons again toggles the function OFF.

To deactivate the Ortho Line Function, type:

MOUSE +0   (Switches the Ortho Line Function OFF)

E. Multiple Parameters

------------------

It is also possible to select multiple parameters in one command.

The sequence for selecting multiple parameters is:

1). Acceleration (*a)

2). Ortho Line function (+s)

3). COM port (p or -i,q)

Where a is from 1 to 5, s is 0 or 1, p is from 1 to 4, i is the

I/O Port Address, and q is the IRQ No.

Ex: A >MOUSE *4+1-3E8,4

This case allows for faster movement of the cursor (*4) on

the screen and the mouse port address is 3E8H which IRQ4 is ass-

igned. We also activated the Ortho line Function (+1).

Ex: A >MOUSE *3-279

Acceleration Setting 3, I/O Port Address 279H, and default

IRQ no. 5.

Ex: A >MOUSE +1-260

The Ortho line function is activated and we set the I/O port

address to 260H. The IRQ would be the default, no. 5.

Ex: A >MOUSE 4*2

This command connects your mouse to communication port COM4

and uses the default address and IRQ no. associated with COM 4

(refer to the table in section A of this file), and allows for

slower movement of the cursor (*2) on the screen.

F. The Dynamic Mouse Driver V9.10 provides a bus mouse interface which

is suitable for Microsoft Bus Mouse, Microsoft InPort Mouse.

Important: This README.DOC takes precedence over the user's

manual that accompanies your input device (mouse

or trackball). The Dynamic Mouse driver V9.10 is the

latest driver currently available. The driver is

continually updated and if you're trying to run

a new software program and having problems using

the input device, please contact the place of

purchase for assistance.